UFAMeasy Machine Server Technical Report

Multi-Machine Scaling, Ingestion Topology, WAN/Cloud Dynamics & Cost Analysis

# 1. Executive Summary & System Overview

The UFAMeasy Machine Server is an Industrial Internet of Things (IIoT) backend and real-time monitoring dashboard tailored for 5-axis Directed Energy Deposition (DED) / Laser Additive Manufacturing systems.

The server bridges physical manufacturing hardware (laser heads, powder feeders, inert gas regulators, motion controllers) and CAD/CAM software (FreeCAD UFAMeasy Workbench) with web-based operational monitoring interfaces.

## Core Technology Stack

Ingestion Protocol: MQTT (Mosquitto Broker) for low-latency, event-driven, decoupled telemetry.

Backend Application: Python 3.13+ with FastAPI and Uvicorn (asynchronous ASGI event loop).

In-Memory Cache: Python-native thread-safe StateStore (lock-protected parameter & slice snapshot cache).

Relational Persistence: SQLite database (data/params.db) recording device registries, manufacturing sessions, slice metadata, and high-frequency runtime logs.

Real-Time Client Streaming: Asynchronous WebSockets (/ws) broadcasting state deltas to connected browser clients.

Frontend Dashboard: Vanilla JavaScript single-page application (zero build-step dependency), supporting live G-code tracking, multi-axis positions, sensor gauges, and video/log inspection.

UFAMeasy (FreeCAD App) ──► core/mqtt_publisher.py ──► Mosquitto (Port 1883/8883) │ server/mqtt_client.py ┌────────┴────────┐ ▼ ▼ server/db.py state_store.py │ ws_manager.py │ ui/index.html (Browser)

# 2. Multi-Machine Handling Architecture (Example: 3 Machines)

When scaling from a single workshop machine to multiple industrial cells (e.g., Machine 1: device_001, Machine 2: device_002, and Machine 3: device_003), the server handles routing, isolation, storage, and presentation through clear architectural boundaries.

## 2.1 Device Identification & MQTT Topic Namespacing

Every machine runs its own instance of the UFAMeasy FreeCAD client or controller agent. Each machine is assigned a unique identifier (device_id). The server organizes message streams using hierarchical MQTT topics:

ufameasy/{device_id}/{subtopic}

| Telemetry Topic | Machine 1 (device_001) | Machine 2 (device_002) | Machine 3 (device_003) |
| --- | --- | --- | --- |
| Session Start | ufameasy/device_001/session/start | ufameasy/device_002/session/start | ufameasy/device_003/session/start |
| Runtime Telemetry | ufameasy/device_001/runtime | ufameasy/device_002/runtime | ufameasy/device_003/runtime |
| Axis Positions | ufameasy/device_001/runtime/position | ufameasy/device_002/runtime/position | ufameasy/device_003/runtime/position |
| Slice Snapshot | ufameasy/device_001/slice/{n} | ufameasy/device_002/slice/{n} | ufameasy/device_003/slice/{n} |
| Estimate Request | ufameasy/device_001/estimate/request | ufameasy/device_002/estimate/request | ufameasy/device_003/estimate/request |
| Session End | ufameasy/device_001/session/end | ufameasy/device_002/session/end | ufameasy/device_003/session/end |

## Ingestion Routing in server/mqtt_client.py

The server subscribes once to the wildcard topic 'ufameasy/#'. Upon receiving a packet, it splits the topic string, extracts the device_id token in O(1) time, and routes updates directly into device-isolated state stores:

topic_parts = msg.topic.split("/") if len(topic_parts) >= 3 and topic_parts[0] == "ufameasy": device_id = topic_parts[1] subtopic = topic_parts[2:] if subtopic == ["runtime"]: # Updates only this device's memory partition and database records state.update_parameter(device_id, key, value) _broadcast({"type": "runtime_update", "device_id": device_id, "data": data})

## 2.2 Data Partitioning & Storage Isolation

1. In-Memory State Store (server/state_store.py): Dictionaries are partitioned strictly by device_id. Mutex locks (threading.Lock) prevent race conditions when multiple machines stream runtime updates at 10-50 Hz concurrently.

2. Database Schema Isolation (server/db.py): The relational schema enforces strict foreign-key integrity. The 'devices' table maps device_id. The 'sessions' table indexes every print run by session_id and device_id. Sub-tables ('runtime_log' and 'slice_data') reference session_id, ensuring queries for Machine 1 never scan or lock data belonging to Machine 2 or 3.

## 2.3 Dashboard Experience with 3 Machines

In the frontend UI (ui/index.html), a device selector (<select id='deviceSelect'>) dynamically scopes the dashboard view. When an operator selects Machine 2, the UI refreshes live 5-axis coordinates (X, Y, Z, A, C), process gauges (Laser Wattage, Powder Feed Rate, Shielding Gas), session history, and camera feeds specifically for Machine 2. WebSocket packets are broadcast with a device_id envelope, enabling background machine status tracking without interrupting the active screen.

# 3. Network Architecture: Within Network (LAN) vs. Outside Network (WAN/Cloud)

Handling 3 machines depends directly on where the server sits relative to the physical manufacturing machines.

| Architectural Criterion | Scenario A: Within Network (LAN) | Scenario B: Outside Network (Cloud) |
| --- | --- | --- |
| Server Host Location | On-Premises Local Host (192.168.1.100) | Public Cloud VPS (AWS, DigitalOcean, Hetzner) |
| MQTT Telemetry Port | Port 1883 (Plain TCP, Local Subnet) | Port 8883 (MQTTS with TLS v1.3 Encryption) |
| Addressing & Ingress | RFC 1918 Private IP addresses | Public FQDN / Elastic IP with outbound NAT traversal |
| Diagnostic Logs (FTP) | Direct pull from machine FTP (Port 2121) | Push model: Machine uploads HTTP POST/S3 directly |
| Live Video Streaming | Direct HTTP stream from IP:8765 | Outbound WebRTC / MediaMTX relay or Mesh VPN |
| Network Infrastructure Cost | $0 (Uses local factory switch/Ethernet) | Cloud subscription + object storage ($25-$50/mo) |

## The Core Cloud Networking Challenge: Inbound vs. Outbound NAT Traversal

1. MQTT Telemetry Works Seamlessly Out-of-the-Box: MQTT connections are outbound. Machines inside the factory initiate outbound TCP connections to 'mqtt.yourcompany.com:8883'. Standard factory firewalls permit outbound traffic by default, creating dynamic NAT state table entries. No static public IP or port forwarding is required on the factory router.

2. Direct Server-to-Machine Connections Fail Without Architecture Changes: In the current local code, the server initiates connections inward to machine IPs (ftp.connect(machine_ip, 2121) and socket probes on 8765). A cloud server cannot dial private IP addresses (e.g. 192.168.1.10) inside a factory network.

## 3 Architectural Solutions for Cloud Connectivity

| Pattern | Technical Mechanism | Recommended Use Case |
| --- | --- | --- |
| Pattern 1: Reverse Push (Best Practice) | Machines push logs and recordings to Cloud Object Storage (S3/R2) or via HTTP POST /recordings/upload. | Modern, secure production. Zero VPN maintenance required. |
| Pattern 2: Industrial Mesh VPN (Tailscale/WireGuard) | Lightweight VPN agents on all machines and cloud server create an encrypted virtual subnet (100.64.0.x). | Fastest migration without rewriting existing FTP or camera routes. |
| Pattern 3: Edge Streaming Gateway (WebRTC/MediaMTX) | Edge machines push RTSP/WebRTC streams outbound to a cloud media gateway. | Ultra-low-latency camera feeds for multiple simultaneous viewers. |

# 4. Security Requirements for Cloud Deployment

Deploying on the public internet requires four mandatory security controls:

TLS / SSL Cryptographic Encryption: Enforce MQTTS over Port 8883 with TLS v1.2/1.3. Enforce HTTPS/WSS on Port 443 via an Nginx reverse proxy with automated Let's Encrypt certificates.

MQTT Authentication & Topic-Level ACLs: Disable anonymous broker access. Assign unique credentials per machine (e.g. device_001 can only publish to ufameasy/device_001/#; backend server subscribes to ufameasy/#).

API & WebSocket Authentication: Protect FastAPI REST endpoints and WebSocket handshakes (/ws?token=...) using signed JWT tokens or API keys.

Database Engine Upgrade (PostgreSQL): While SQLite is effective for local single-node execution, concurrent writes from multiple high-frequency industrial machines over WAN warrant migrating server/db.py to PostgreSQL to eliminate database lock contention.

# 5. Estimated Cloud Deployment Costs

The following financial matrix models the infrastructure required to support 3 to 10 concurrently streaming industrial additive manufacturing machines:

| Cost Category | Tier 1: Pilot / Testing (Single Cloud VPS) | Tier 2: Standard Production (VPS + S3 + Backups) | Tier 3: Enterprise Cloud (AWS Multi-AZ Managed) |
| --- | --- | --- | --- |
| Target Capacity | 1 - 3 Machines (R&D) | 3 - 10 Machines (Recommended) | 10 - 50 Machines (High Availability) |
| Compute Instance | $12 - $18 / mo (DigitalOcean / Hetzner 2 vCPU, 4GB RAM) | $20 - $28 / mo (AWS Lightsail / DO Droplet 4GB RAM) | $70 - $95 / mo (AWS EC2 t4g.medium / ECS Fargate) |
| Database Layer | $0.00 (Embedded SQLite on SSD) | $15.00 / mo (Managed DB or automated S3 snapshots) | $60 - $80 / mo (AWS RDS PostgreSQL Multi-AZ) |
| MQTT Broker (Mosquitto) | $0.00 (Co-located Mosquitto on VPS) | $0.00 (Docker container on same VPS) | $25 - $40 / mo (AWS IoT Core / Managed EMQX) |
| Object Storage (Recordings) | $0.00 (Stored on local VPS disk) | $5.00 / mo (Cloudflare R2 / AWS S3, ~250 GB) | $15 - $25 / mo (AWS S3 Standard + Lifecycle) |
| Egress Bandwidth | $0.00 (Within 2 - 4 TB VPS quota) | $0 - $5 / mo (Cloudflare R2 has $0 egress fees) | $10 - $20 / mo (AWS Internet Data Transfer) |
| Domain & SSL Certificates | $1.00 / mo (~$12/year domain, Let's Encrypt $0) | $1.00 / mo (~$12/year domain, Let's Encrypt $0) | $1.00 / mo (Route53 + ACM free certs) |

[RECOMMENDED CLOUD SPECIFICATION] Recommended Strategy for UFAMeasy: Deploy Tier 2 on a single 2 vCPU / 4 GB RAM Cloud VPS (AWS Lightsail or DigitalOcean Droplet) costing ~$20-$25/month. Run Mosquitto (MQTTS 8883) and FastAPI (HTTPS 443 via Nginx) co-located on the same instance ($0 extra broker cost). Use Cloudflare R2 for storing video recordings and diagnostic logs ($5/month with $0 egress fees). Total estimated production budget: $25 to $35 / month (~₹2,100 to ₹3,000 INR/month).

## Telemetry Bandwidth Sizing Calculation

• Raw Telemetry Payload: ~250 bytes per JSON message
• Transmission Frequency: 5 Hz per machine = 1.25 KB/sec
• Cumulative Data Rate (3 Machines): 3.75 KB/sec
• Monthly Telemetry Volume (8 hrs/day, 22 days/mo): ~2.37 GB/month
• Conclusion: Telemetry data consumption is negligible and easily accommodated within standard VPS bandwidth quotas (1,000 to 2,000 GB/month).

# 6. Multi-Machine Readiness Checklist

Client Namespacing: Configure device_id (device_001, device_002, device_003) in each FreeCAD client profile.

Broker Security: Configure Mosquitto listener 8883 with TLS certificate paths and password authentication (mosquitto_passwd).

Firewall Rules: Expose external ports 443 (HTTPS/WSS) and 8883 (MQTTS); bind ports 8000 (FastAPI) and 1883 (internal MQTT) strictly to localhost (127.0.0.1).

Log & Recording Push: Replace inbound FTP polling in server/routes.py with client-initiated HTTP POST multipart uploads into Cloudflare R2 / S3.

Database Migration: Migrate server/db.py to PostgreSQL using SQLAlchemy + asyncpg when scaling beyond 5 concurrent machines.